- ES-10*– Free standing, emergency shower with ABS plastic shower head (optional stainless steel shower head “SSH”), internal 20 GPM flow control, 1 ¼” IPS Schedule 40 hot dipped galvanized pipe and fittings, 1” IPS chrome-plated brass stay-open ball valve, stainless steel actuating arm, polished stainless steel pull rod, and powder-coated cast aluminum floor flange.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be hydrostatically tested to meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1662 and G1662SSH