- EF-22* – Deck mounted, eye/face wash with stainless steel bowl, (optional ABS plastic bowl “P”), ½” U.S. made chrome-plated brass stay-open ball valve, powder-coated cast aluminum flag handle, and 1¼” mounting shank. Unit shall have (2) polypropylene FS-Plus™ spray heads with integral “flip-top” dust covers, filters and 3.2 GPM flow control orifices mounted on a chrome-plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1755 and G1755P