- EF-25* – Deck mounted, eye/face wash with recessed, flanged stainless steel bowl, ½” U.S. made brass stay-open ball valve, push down valve activation, deck flange, and mounting hardware. Unit shall have (2) polypropylene FS-Plus™ spray heads with integral “flip-top” dust covers, filters and 3.2 GPM flow control orifices mounted on a chrome-plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1765