- EF-31* – Deck mounted AutoFlow™ swing-down eye/face wash less bowl with ½” IPS plug-type valve with PTFE coated O-ring seals and stainless steel enclosure. Supplied with in-line strainer to protect valve and spray heads from debris in water line. Unit shall have (2) polypropylene FS-Plus™ spray heads with integral “flip-top” dust covers, filters and 3.2 GPM flow control orifices mounted on a chrome-plated brass eyewash assembly (Specify spray head configuration).  Activate valve by rotating 90° from stored position.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1779