- EW-2* – Right hand (specify left hand mounting “LH”) deck mounted 90° swivel eyewash less bowl with ½” U.S. made stay-open brass ball valve and powder coated cast aluminum flag handle. Unit shall have (2) polypropylene GS-Plus™ spray heads with integral “flip-top” dust covers, filters and 1.6 GPM flow control orifices mounted on a chrome plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1806 and G1806LH