- EW-3* – Deck mounted eyewash with recessed, flanged stainless steel bowl, ½” U.S. made chrome plated brass stay open ball valve, powder coated cast aluminum flag handle, and mounting hardware. Unit shall have (2) polypropylene GS-Plus™ spray heads with integral “flip-top” dust covers, filters and 1.6 GPM flow control orifices mounted on a chrome plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1808