- EW-15* – Pedestal mounted eyewash with ABS plastic bowl, Schedule 40 galvanized pipe and fittings, ½” U.S. made chrome-plated stay-open ball valve, powder-coated cast aluminum flag handle and floor flange.  Unit shall have (2) polypropylene GS-Plus™ spray heads with integral “flip-top” dust covers, filters and 1.6 GPM flow control orifices mounted on a chrome-plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be hydrostatically tested to meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1825P