- SS-4* - Combination eyewash and shower safety station with ABS plastic shower head (optional stainless steel shower head “SSH”), internal 20 GPM flow control, stainless steel eyewash bowl, powder-coated cast aluminum flag handle and floor flange, 1¼” IPS Schedule 40 galvanized pipe and fittings, 1” IPS and ½” IPS U.S. made chrome-plated brass stay-open ball valves, and polished stainless steel pull rod.  Unit shall have (2) polypropylene GS-Plus™ spray heads with integral “flip-top” dust covers, filters, and 1.6 GPM flow control orifices mounted on a chrome-plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Drench Hose: G5011 with 8ft. PVC hose

- Performance:  Unit shall meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1902HS and G1902HS-SSH