- SS-25* - Combination eye/face wash and shower safety station with ABS plastic shower head (optional stainless steel shower head “SSH”), internal 20 GPM flow control, stainless steel eye/face wash bowl, powder-coated cast aluminum flag handle and floor flange, 1¼” IPS Schedule 40 galvanized pipe and fittings, 1” IPS and ½” IPS U.S. made chrome-plated brass stay-open ball valves, and polished stainless steel pull rod.  Unit shall have (2) polypropylene FS-Plus™ spray heads with integral “flip-top” dust covers, filters, and 3.2 GPM flow control orifices mounted on a chrome-plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1950 and G1950SSH