- SS-30* - Combination eye/face wash and shower safety station.  Unit is constructed of polished chrome-plated brass pipe and fittings. 1” IPS and ½” IPS U.S. made polished chrome-plated brass stay-open ball valves, polished stainless steel pull rod, stainless steel shower head with 20 GPM flow control, and stainless steel eye/face wash bowl. Unit shall have (2) polypropylene FS-Plus™ spray heads with integral “flip-top” dust covers, filters, and 3.2 GPM flow control orifices mounted on a chrome-plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1950PCC