- SS-32* - Corrosion resistant, all stainless steel construction, combination eyewash and shower safety station with stainless steel shower head, internal 20 GPM flow control, stainless steel eyewash bowl, stainless steel flag handle and floor flange, 1¼” IPS Schedule 40 stainless steel pipe and fittings, 1” IPS and ½” IPS U.S. made stainless steel stay-open ball valves, and polished stainless steel pull rod.  Unit shall have (2) polypropylene GS-Plus™ spray heads with integral “flip-top” dust covers, filters, and 1.6 GPM flow control orifices mounted on a stainless steel head assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1991