- SS-35* - Corrosion resistant, combination eye/face wash and shower safety station with ABS plastic shower head, internal 20 GPM flow control, ABS plastic eye/face wash bowl, cast aluminum powder-coated flag handle, PVC floor flange, 2” IPS Schedule 80 PVC pipe and fittings, 1” IPS and ½” IPS Type 316 stainless steel stay-open valve. Valve has stainless steel ball valves and PTFE seals, and polished stainless steel pull rod.  Unit shall have (2) polypropylene FS-Plus™ spray heads with integral “flip-top” dust covers, filters, and 3.2 GPM flow control orifices mounted on a PVC-coated brass head assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1993