- SS-43* - Corrosion resistant, type 316 all stainless steel construction, combination eye/face wash and shower safety station with stainless steel shower head, internal 20 GPM flow control, stainless steel eye/face wash bowl, and stainless steel flag handle and floor flange. 1¼” IPS Schedule 40 type 316 stainless steel pipe and fittings, 1” IPS and ½” IPS U.S. made stainless steel stay-open ball valves, and polished stainless steel pull rod.  Unit shall have (2) polypropylene FS-Plus™ spray heads with integral “flip-top” dust covers, filters, and 3.2 GPM flow control orifices mounted on a stainless steel head assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1996