- DH-4* - Wall mounted, hand-held drench hose with ½” IPS U.S. made chrome-plated brass squeeze valve with replaceable stainless steel seat (optional locking clip, “LC”), stainless steel squeeze handle with plastic cover, nylon handle, powder-coated mounting bracket, and 12ft. nylon coiled hose.  Unit shall have polypropylene FS-Plus™ spray head with integral “flip-top” dust cover, filter and 3.2 GPM flow control orifice angled at 45°.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G5016 and G5016LC