- DH-6* - Deck mounted, hand-held drench hose with ½” IPS U.S. made chrome-plated brass squeeze valve with replaceable stainless steel seat (optional locking clip, “LC”), stainless steel squeeze handle with plastic cover, nylon handle, nylon deck flange, and 8ft. PVC hose. Unit shall have polypropylene FS-Plus™ spray head with integral “flip-top” dust cover, filter and 3.2 GPM flow control orifice.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G5020 and G5020LC