- DH-13* - Wall mounted, hand-held eyewash/drench hose with ½” IPS U.S. made chrome-plated brass squeeze valve with replaceable stainless steel seat and locking clip, stainless steel squeeze handle with plastic cover, nylon handle, powder-coated wall bracket, and 8ft. PVC hose.  Unit shall have (2) polypropylene GS-Plus™ spray heads with integral “flip-top” dust covers, filters and 1.6 GPM flow control orifices mounted on a chrome-plated brass eyewash assembly.  Unit shall include ANSI compliant sign

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G5026