- EFBFVR-7* – Barrier Free and vandal resistant, wall mounted eye/face wash with stainless steel bowl, stainless steel wrap-around skirt, ½” U.S. made chrome-plated brass stay-open ball valve, powder-coated cast aluminum flag handle.  Unit shall have (4) chrome-plated brass GS-Plus™ spray heads with integral stainless steel “flip-top” dust covers, filters, and 1.6 GPM flow control orifices mounted on a chrome-plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Vandal Resistance: All connections shall be secured with setscrews or cement to prevent disassembly.

- Performance:  Unit complies with ADA requirements for accessibility by handicapped persons. Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment GBFVR1721