- SSBFSC-9* – Barrier Free, recessed, wall mounted combination eye/face wash and shower safety station for standard 3-5/8” wall with matching fire extinguisher housing, recessed mounted shower head, patented stainless steel shower-actuating arm and swing-down stainless steel combination eye/face wash drain pan.  Unit construction shall be welded 16-gauge type 304 stainless steel with #4 brushed satin finish. Unit shall include stainless steel flanged shower head, internal 20 GPM flow control, chrome-plated brass eye/face wash supply fittings, brass unions, and U.S made full-port brass ball valve for shower. Eye/face wash valve shall be AutoFlow™, plug-type design with PTFE coated O-rings to seal valve orifices, polypropylene FS-Plus™ spray heads with individual 3.2 GPM flow controls and polyurethane filters. Supplied with in-line strainer to protect eye/face wash valve and spray heads from debris in water line, and 2” IPS drain. Activate eye/face wash valve by rotating 90° from stored position. Unit shall include ANSI compliant sign.

- Fire Extinguisher Housing: Accommodates Oval brand fire extinguisher model 10JABC. (Extinguisher sold separately)

- Performance: Unit complies with ADA requirements for accessibility by handicapped persons. Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture: Guardian GSC2660B