- SSVR-9* - Combination eyewash and shower safety station with chrome-plated brass shower head, internal 20 GPM flow control, stainless steel eyewash bowl, powder-coated cast aluminum flag handle and floor flange, 1¼” IPS Schedule 40 galvanized pipe and fittings, 1” IPS U.S. made chrome-plated brass self-closing ball valve, ½” IPS U.S. made self-closing chrome-plated ball valve, and polished stainless steel pull rod.  Unit shall have (2) chrome-plated brass GS-Plus™ spray heads with integral stainless steel “flip-top” dust covers, filters, and 1.6 GPM flow control orifices mounted on a chrome-plated brass eyewash assembly. Unit shall include ANSI compliant sign.

- Vandal Resistance: All connections shall be secured with set screws or cement to prevent disassembly.  Performance:  Unit shall meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment GVR1903SC