- DHVR-8* - Deck mounted, hand-held drench hose with ½” IPS U.S. made chrome-plated brass squeeze valve with replaceable stainless steel seat (optional locking clip, “LC”), nylon handle, stainless steel squeeze handle with plastic cover, nylon deck flange, and 8ft. PVC hose.  Unit shall have chrome-plated brass FS-Plus™ spray head with integral stainless steel “flip-top” dust cover, filter and 3.2 GPM flow control orifice. Unit shall include ANSI compliant sign.

- Vandal Resistance: All connections shall be secured with set screws or cement to prevent disassembly.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested before delivery to meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment GVR5020 and GVR5020LC