- DHVR-10* - Deck mounted, hand-held eyewash/drench hose with ½” IPS U.S. made chrome-plated brass squeeze valve with replaceable stainless steel seat and locking clip, nylon handle, stainless steel squeeze handle with plastic cover, nylon deck flange with locator guide, and 8ft. PVC hose.  Unit shall have (2) chrome-plated brass GS-Plus™ spray heads with integral stainless steel “flip-top” dust covers, filters and 1.6 GPM flow control orifices mounted on a chrome-plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Vandal Resistance: All connections shall be secured with set screws or cement to prevent disassembly.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested before delivery to meet or exceed ANSI Z358.1 – 2014, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment GVR5022